Automatic imaging for colony assay

- Turn on the power of three instruments: Power Supply 232, X-Cite 120LEDBoost, microscope

- Turn on ZEN blue on desktop

- Small window will pop out showing “Stage/Focus is not Calibrated” -> Calibrate Now: automatic calibration

- Import saved image for coping the previous settings:

- File -> Open -> DATA (D:) -> select file -> Open

- Acquisition -> Reuse

- Alternative way- import saved template: Acquisition -> the default is MultiChannel Experiment*, select the desired template

- Check Software Autofocus -> select Smart (not Full)

- Check Channels: only the Bright field is ON

- Tiles -> Sample Carrier ->

- Select Template: select Petri Dish 35 mm -> OK

- Calibrate -> Sample Carrier Calibration Wizard:

- 1/4 Select Low Magnification Objective: Objective: 10x Air 0.3 -> Next

- 2/4 Setup Illumination: make sure that Transmitted light ON/Reflected light OFF -> Next ->

- 3/4 Calibration Stage: X/Y Position: click Calibrate -> Continue -> Next ->

- 4/4 Search Reference Point: manually locate dish at the center of imaging field -> look at little screen attached to microscope: click Make it Visible! -> the light will shed on the dish -> adjust the position of the dish OR the value of Position X/ Position Y (25000 μm/ 40,000 μm), make the light shed at the center of the dish -> click Set Current X/Y -> Finish

- Draw tiles

Tiles -> Tile Regions -> Contour: select rectangular/circular contour shape -> Add “+ “ -> adjust the size of tiles (yellow boxes)

- Check Tiles -> Options -> Tile Overlap: 10%

- Acquire images:

- Tiles -> Advanced Setup -> select desired tiles from drop-down menu

- Live: to get live image, zoom in and adjust the focus and brightness (make the median of brightness peak locate at the middle of X axis)

- Start experiment -> Save

- Reduce the volume of diaminobenzidine solution from 1 ml to 0.5 ml helps improving the resolution

Image Stitching

- Processing -> Batch -> Batch Method -> Stitching -> + Add Files

- Check Parameters:

- Edge Detector: Yes

- Minimal Overlap: 5%

- Maximal Shift: 10%

- Comparer: Optimized

- Global Optimizer:  Best

- Click Apply

- The best stitching is the combination of the following parameters: stitching + fuse + correct shading edge